Case description - Quico

QuiCo is the world market leader in the production of quibi (plural of quibus). Notwithstanding the secrecy surrounding the quibus, we have been informed that the production process depends mainly on the length and the width (both in cm).

Quibi are produced according to customer orders. Each order mentions the number of quibi, the type of quibus using the article number (a four-digit code) and the agreed delivery date. The following articles and their characteristics are known:

| Article No. | Length | Width |
| --- | --- | --- |
| 6717 | 120 cm | 6.7 cm |
| 7818 | 130 cm | 56 cm |
| 8040 | 80 cm | 4 cm |
| 8170 | 80 cm | 17 cm |
| 8345 | 110 cm | 3.4 cm |
| 8500 | 80 cm | 50 cm |
| 9010 | 100 cm | 34.2 cm |
| 9046 | 90 cm | 4.6 cm |
| 9059 | 90 cm | 5.9 cm |

The production process consists of three operations: cutting, folding and packaging. Each operation can be produced on one (or more) machine(s). An operation has to be completely finished before the next operation can be started and a period of 10 minutes is required to transport the order from one machine to the next.

There are two cutting machines (CM1 and CM2), three folding machines (FM-IV,-FM-IX and FM-X) and one packaging machine (PM) in QuiCo. The older model FM-IV can only fold quibi less than 45 cm wide.

The details about the machines are shown in the table below:

| Machine | Maximum width (cm) | Speed (cm/min) |
| --- | --- | --- |
| CM1 | 9999 | 250 |
| CM2 | 9999 | 340 |
| FM-IV | 45 | 250 |
| FM-IX | 9999 | 480 |
| FM-X | 9999 | 300 |
| PM | 9999 | 550 |

The setup time for cutting for a certain order depends on the difference in length between that order and the previous order on the same machine. The following table lists the duration of the setup time required:

| Machine | Length difference (cm) | Duration |
| --- | --- | --- |
| CM1 | 0 – 9 | 0:10:00 |
| CM1 | 10 – 20 | 0:25:00 |
| CM1 | 21 – 9999 | 1:05:00 |
| CM2 | 0 – 9 | 0:10:00 |
| CM2 | 10 – 20 | 0:45:00 |
| CM2 | 21 – 9999 | 1:00:00 |

All machines not listed in this table are assumed not to have any setup time.
QuiCo only produces during weekdays, that is from Monday to Friday, from 6:00 until 18:00. If an operation is started but cannot be finished before 18:00, the machine is halted. During the next working day, no special action is needed before restarting the machine.

QuiCo aims for maximum output. It tries to achieve maximal machine efficiency: minimal idle and setup time (without changing delivery performance). Customers are asked to order 1,000 quibi or more.